RV TOTAL CONTROL

Project Reference Document

Consolidated from session logs: May 26 – June 10, 2026

Steve Bradshaw  |  ve7cbh  |  Nanaimo, BC

github.com/ve7cbh/RV-total-control

Current Status Phase 2 Complete · Phase 3 Active Core Docker Stack Running · Dual RTL-SDR Live · Waveshare RS-485 Gateway In Hand

# 1  Project Overview

RV Total Control (RVTC) is a full-stack monitoring and control system for a recreational vehicle, built on a Beelink J45 mini-PC running Linux Mint LMDE. The system integrates weather sensing, power management (solar, inverter, grid, generator), tank monitoring, water quality measurement, and location tracking into a unified dashboard accessible both locally and over the internet.

The architecture follows Ansible-managed Docker containers feeding data via MQTT into InfluxDB and Grafana, with Home Assistant for automation and alerting. Engineering standards follow navy-grade practices throughout: twisted-pair shielded cable, 4–20 mA current loops for long analog runs, RS-485 Modbus RTU for digital sensor buses, and single-point grounding at all enclosures.

## 1.1  Phase Status

| Phase | Title | Status | Notes |
| --- | --- | --- | --- |
| 0 | Architecture & Design | ✅ Complete |  |
| 1 | Beelink J45 Build | ✅ Complete | OS, Docker, Ansible, Git |
| 2 | Core Stack Deployment | ✅ Complete | Full stack live as of 2026-06-09 |
| 3 | Power Integration | 🔄 Active | Waveshare gateway in hand |
| 4 | Tank & Propane Sensing | ⏳ Pending |  |
| 5 | Water Monitoring | ⏳ Pending | Solsante V1.5 subset |
| 6 | Baseline & Handover | ⏳ Pending |  |
| 7 | Sensor Fusion | ⏳ Architecture phase | Python fusion service |

# 2  Hardware

## 2.1  Beelink J45 (Control Node)

| Item | Specification |
| --- | --- |
| Host | Beelink J45 |
| CPU | Intel Pentium J4205 (4-core) |
| RAM | 8 GB |
| Root drive | /dev/sda — 256 GB — mounted / |
| Data drive | /dev/sdb — 640 GB — mounted /data |
| OS | Linux Mint LMDE (Debian trixie base) |
| Hostname | ve7cbh-control |
| Primary user | ve7cbh |
| Access | SSH from Windows workstation (192.168.88.2) |
| IP | 192.168.88.3 (ethernet, enp1s0) — WiFi disabled |

## 2.2  Software on J45

| Package | Version |
| --- | --- |
| Docker CE | Latest (Compose v5.1.4) |
| Ansible | core 2.19.4 |
| Python | 3.13.5 |
| Git | 2.47.3 |
| rtl-433 | Latest (host + Docker container) |
| rtl-sdr | Latest (provides rtl_eeprom, udev rules) |
| mosquitto-clients | Latest (mosquitto_sub / mosquitto_pub CLI) |
| sqlite3 | Latest (host — WeeWX archive queries) |

## 2.3  RTL-SDR Dongles

| Unit | Tuner | Serial | Container | Status |
| --- | --- | --- | --- | --- |
| RTL-SDR Blog V3 (aluminium, SMA) | R820T | 1024 | rtl433 (primary) | ✅ Active |
| Clone (DVB-T, R828D — higher spec tuner despite clone status) | R828D | 00000001 | rtl433b (secondary) | ✅ Active |

- Both containers pin to dongle via -d <serial> — immune to device index changes on reboot.

- USB path in compose: /dev/bus/usb:/dev/bus/usb (whole bus passthrough).

- PLL not locked warning is benign — always present on both tuners.

- Counterintuitively the R828D (generally higher spec) is in the clone, not the Blog V3.

## 2.4  Acurite 5n1 Weather Stations

| ID | Channel | Location | Status |
| --- | --- | --- | --- |
| 1111 | A | Home base — pole-mounted | Active — primary |
| 291 | C | Mobile — travels with RV | At home — filtered out in weewx.conf |
| Spare | — | — | DOA (HW-12) — replacement pending |

## 2.5  Modbus / RS-485 Gateway

Waveshare 8-Ch RS485 to RJ45 Ethernet Serial Server (ASIN B0F5WXX4ZQ). Industrial isolation, PoE, Modbus RTU/TCP, MQTT gateway. Status: In hand — commissioning next session (HW-01).

| Port | TCP | Bus / Purpose | Device(s) | Phase |
| --- | --- | --- | --- | --- |
| RS-485/1 | 4001 | Power — Solar | EPEVER MPPT60 | 3 |
| RS-485/2 | 4002 | Power — Inverter | SAMLUX EVO-2212 | 3 |
| RS-485/3 | 4003 | Power — Grid meter | KWS-303L (grid) | 3 |
| RS-485/4 | 4004 | Power — Generator | KWS-303L (generator) | 3 |
| RS-485/5 | 4005 | Water sensors | Pressure + Filter ΔP + Turbidity | 5 |
| RS-485/6 | 4006 | GNSS | E108-GN03G-485 position/time receiver | 3 |
| RS-485/7 | 4007 | Spare | — | — |
| RS-485/8 | 4008 | Spare | — | — |

# 3  Network

## 3.1  IP Allocation

| IP | Device | Notes |
| --- | --- | --- |
| 192.168.88.1 | MikroTik gateway | Primary router |
| 192.168.88.2 | Windows workstation | SSH client |
| 192.168.88.3 | Beelink J45 (enp1s0) | All services — primary IP |
| 192.168.88.4 | J45 WiFi (wlp3s0) | Disabled — autoconnect=no, radio off |
| 192.168.88.5 | Waveshare RS-485 gateway | Pending commissioning |

## 3.2  Pi-hole DNS (.lan records)

All local DNS records use .lan TLD. The .local TLD was abandoned due to mDNS/Avahi conflicts. Always use http:// prefix — bare .lan names are intercepted as search queries.

| Domain | IP | Notes |
| --- | --- | --- |
| weewx.lan | 192.168.88.3 | nginx reverse proxy — Belchertown skin |
| grafana.lan | 192.168.88.3 | nginx reverse proxy |
| influxdb.lan | 192.168.88.3 | nginx reverse proxy |
| homeassistant.lan | 192.168.88.3 | nginx reverse proxy |
| pihole.lan | 192.168.88.3 | nginx reverse proxy (no password) |

## 3.3  MikroTik Router

Config file: config/rv-mikrotik-config.rsc (updated 2026-06-09, ether8 + pinhole fixes). DNS: Primary 192.168.88.3 (Pi-hole), secondary 8.8.8.8. DHCP hands out Pi-hole IP to all clients.

External WeeWX access path:

wifi.solsante.com:8080 → club router → MikroTik rogers-wan:80 → dst-nat → 192.168.88.3:80 → nginx → WeeWX

NOTE: New RSC (ether8 + pinhole fixes) not yet installed on physical router — install and test at next visit.

# 4  Docker Stack

CRITICAL: Always run docker compose up/down from ~/RV-total-control/ — this is the canonical compose file location. Running from /data/docker/volumes/ puts containers on the wrong network (volumes_default instead of rvtc_net) and breaks inter-container DNS.

Docker network: rvtc_net  |  Compose file: ~/RV-total-control/docker-compose.yml

## 4.1  Running Containers

| Container | Image | Port(s) | Volume / Notes |
| --- | --- | --- | --- |
| mosquitto | eclipse-mosquitto | 1883 | /data/docker/volumes/mosquitto |
| influxdb | influxdb:2 | 8086 | /data/docker/volumes/influxdb |
| grafana | grafana/grafana | 3000 | /data/docker/volumes/grafana — user: 472:472 |
| rtl433 | hertzg/rtl_433:latest | — | SN 1024 (Blog V3) — primary |
| rtl433b | hertzg/rtl_433:latest | — | SN 00000001 (clone) — secondary |
| weewx | felddy/weewx | — | /data/docker/volumes/weewx |
| nginx | nginx:alpine | 80 | weewx public_html (ro) + nginx.conf |
| homeassistant | ghcr.io/home-assistant/home-assistant | 8123 | /data/docker/volumes/homeassistant |
| pihole | pihole/pihole | 8880/53 | /data/docker/volumes/pihole — listeningMode=all |

## 4.2  rtl_433 Container Configuration

Both containers pass the whole USB bus and pin to their dongle by serial number:

rtl433:  -M si -d 1024 -F "mqtt://mosquitto:1883,retain=1,events=rtl_433/rtl433/events"

rtl433b: -M si -d 00000001 -F "mqtt://mosquitto:1883,retain=1,events=rtl_433/rtl433b/events"

- WeeWX subscribes to rtl_433/+/events — wildcard receives from both containers.

- Both containers decode the same sensors — WeeWX receives duplicate packets. Benign; deduplication is a Phase 7 concern.

- + wildcard is valid in MQTT subscribe but NOT in publish topics.

- -M si flag does NOT convert Acurite 5n1 field names — they always publish temperature_F and rain_in.

## 4.3  Grafana Notes

- Dashboard lost after reboot 2026-06-09 due to volume permissions issue.

- Fix applied: user: "472:472" in docker-compose.yml. If dashboard missing after restart: sudo chown -R 472:472 /data/docker/volumes/grafana

- Dashboard needs rebuilding (OI-16).

# 5  WeeWX Configuration

## 5.1  Key Settings

| Setting | Value |
| --- | --- |
| Config (host) | /data/docker/volumes/weewx/weewx.conf |
| Config (container) | /data/weewx.conf |
| Management | Manual — copy committed to config/weewx.conf in repo |
| Driver | MQTTSubscribeDriver — subscribes to rtl_433/+/events |
| Station | ve7cbh, 48.6686N, 123.6002W, 46m |
| Units | METRICWX (°C, mm, m/s) via StdConvert target_unit = METRICWX |
| Archive interval | 2.5 min (150 seconds) |
| Time zone | America/Vancouver |
| Skin | Belchertown (dark mode) — output at /data/public_html/belchertown |
| InfluxDB writer | Custom influxdb2.py at /data/docker/volumes/weewx/bin/user/ |
| InfluxDB target | host=influxdb, org=rvtc, bucket=rvtc, measurement=weewx |
| PyEphem | Installed — civil twilight and extended celestial data available |
| ID filter | filter_out_message_when = 291 (excludes Port Renfrew unit) |

## 5.2  Field Mappings (MQTTSubscribeDriver)

[[[[temperature_F]]]]  → outTemp  (units=degree_F, StdConvert handles F→C)

[[[[humidity]]]]       → outHumidity

[[[[wind_avg_km_h]]]]  → windSpeed  (units=km_per_hour)

[[[[wind_dir_deg]]]]   → windDir

[[[[rain_in]]]]        → rain  (units=inch, contains_total=true)

CRITICAL: contains_total = true is essential for rain. The Acurite 5n1 sends cumulative bucket counts. Without this setting, every reading adds to the total, producing phantom rain accumulation.

## 5.3  Database

Archive DB: /data/docker/volumes/weewx/archive/weewx.sdb (sqlite3 on host — not in container)

Useful queries:

sqlite3 /data/docker/volumes/weewx/archive/weewx.sdb \

"SELECT datetime(dateTime,'unixepoch','localtime') as ts, rain, rainRate FROM archive ORDER BY dateTime DESC LIMIT 20;"

Rebuild daily summaries after archive edits:

docker exec weewx weectl database rebuild-daily --config=/data/weewx.conf --date=YYYY-MM-DD --yes

NOTE: InfluxDB bucket flushed 2026-06-03 — clean metric data from ~15:35 UTC onward only. WeeWX archive retains full history.

## 5.4  Known WeeWX Issues

- OI-32: WeeWX upstream bug — contains_total=true + hardware fault → corrupted archive_day_rain → silent stats failure. Bug report pending.

- HW-14: Rain gauge physical inspection at club still required — sensor behaviour now better understood.

- Rain ID filter (filter_out_message_when = 291) added but not yet tested under live rain conditions.

# 6  Ansible & Repository

## 6.1  Configuration Files

ansible.cfg (repo root):

[defaults]

interpreter_python = /usr/bin/python3.13

vault_password_file = ~/.vault_pass

Vault password: ~/.vault_pass — plain text, chmod 600, never committed.

inventories/production/hosts.ini:

[rvtc]

localhost ansible_connection=local ansible_python_interpreter=/usr/bin/python3.13

## 6.2  group_vars / host_vars

- group_vars/all/all.yml — all non-sensitive variables

- group_vars/all/vault.yml — ansible-vault encrypted, contains:

  - vault_influxdb_user / vault_influxdb_password

  - vault_grafana_user / vault_grafana_password

  - vault_pihole_user / vault_pihole_password

  - vault_influxdb_token

- host_vars/localhost.yml — J45-specific overrides (NOTE: must match inventory hostname 'localhost')

## 6.3  Roles

| Role | Purpose / Notes |
| --- | --- |
| common | OS baseline, Docker idempotency, UFW firewall, WiFi disable, dvb_usb blacklist, rtl-sdr + sqlite3 packages |
| mosquitto | Eclipse Mosquitto MQTT broker |
| influxdb | InfluxDB 2.x time-series database |
| grafana | Grafana dashboards — user 472:472 baked in |
| rtl433 | rtl_433 container — primary and secondary dongle setup |
| weewx | WeeWX weather engine — config managed manually |
| nginx | Reverse proxy for all .lan domains |
| homeassistant | Home Assistant — onboarding and MQTT integration pending (OI-15) |
| pihole | Pi-hole DNS — listeningMode=all required |

## 6.4  Key Paths

| Path | Purpose |
| --- | --- |
| ~/RV-total-control | Ansible project root — ALWAYS run docker compose from here |
| ~/RV-total-control/docker-compose.yml | Canonical compose file |
| ~/.vault_pass | Ansible Vault password — chmod 600, never committed |
| /data/docker/volumes | All Docker volume data |
| /data/docker/volumes/weewx/weewx.conf | WeeWX live config |
| /data/docker/volumes/weewx/bin/user/influxdb2.py | Custom InfluxDB writer |
| /data/docker/volumes/weewx/archive/weewx.sdb | WeeWX SQLite archive |
| /data/docker/volumes/grafana | Grafana data — must be owned 472:472 |
| /etc/udev/rules.d/99-rtlsdr.rules | RTL-SDR udev rules (MODE=0666 override) |
| /etc/modprobe.d/rtlsdr.conf | dvb_usb_rtl28xxu blacklist |
| /etc/apt/sources.list.d/docker.list | Docker repo — trixie hardcoded (not gigi) |

## 6.5  Known Issues

- LMDE Docker repo: $VERSION_CODENAME returns 'gigi' on LMDE — docker.list must hardcode 'trixie'.

- GitHub auth: PAT stored in ~/.git-credentials via credential.helper store.

- Passwordless sudo: /etc/sudoers.d/ve7cbh — required for Ansible become: true.

- host_vars naming: file must be host_vars/localhost.yml to match inventory hostname.

# 7  Open Items & Backlog

## 7.1  Software / Configuration

| ID | Phase | Item | Notes |
| --- | --- | --- | --- |
| OI-15 | 2/3 | Home Assistant onboarding | 🟡 Open — container up, setup wizard + MQTT integration not done |
| OI-16 | 2 | Grafana weather dashboard | 🟡 Open — lost after reboot 2026-06-09; needs rebuilding |
| OI-18 | 3 | ESPHome Ansible role | 🟡 Open — nginx block in place; role to be created |
| OI-19 | 2/3 | MQTT Explorer | 🟡 Open — may be superseded by Phase 7 fusion UI |
| OI-20 | 3 | HA multi-site linking | 🟡 Open — VPN prerequisite |
| OI-21 | 3+ | VOIP / PBX inter-site | 🟡 Open — prerequisite: OI-20 |
| OI-24 | 3 | 120VAC load shedding on solar | 🟡 Open — smart relay + EPEVER MPPT data required |
| OI-25 | 7 | Phase 7 Sensor Fusion | 🟡 Open — Python fusion service, normalised MQTT schema |
| OI-29 | 3 | GNSS-driven WeeWX position | 🟡 Open — prerequisite: HW-10 |
| OI-30 | 3 | RV position display page | 🟡 Open — prerequisite: HW-10, Phase 7 |
| OI-32 | — | WeeWX upstream bug report | 🟡 Open — contains_total=true + hardware fault → silent rain stats failure |
| OI-33 | 3 | Club bridge Pi | 🟡 Open — rtl_433 + WireGuard at club → home J45 hub; prereq: OI-20 |
| OI-34 | 7 | GNSS geofence source inhibit | 🟡 Open — suppress RV 5n1 when at club; prereq: HW-10, OI-33, Phase 7 |

## 7.2  Hardware / Physical Install

| ID | Phase | Item | Notes |
| --- | --- | --- | --- |
| HW-01 | 3 | Install Waveshare RS485 gateway | 🟡 Open — IN HAND — commissioning next session |
| HW-02 | 3 | Build RS-485 cables | 🟡 Open — for SAMLUX EVO-2212 + EPEVER MPPT |
| HW-03 | 3 | Install 4×100W PV panels | 🟡 Open — get solar data flowing before full array |
| HW-04 | 3 | Wire 9 PV panels (3S×3P ~36V) | 🟡 Open — complete solar system |
| HW-05 | 3/5 | Source barometric pressure sensor | 🟡 Open — for ESP32 sensor node |
| HW-06 | 5 | Build ESP32 sensor node | 🟡 Open — pulse water meter, turbidity, pressure ×2, flow |
| HW-07 | 4/5 | Design tank monitoring sensors | 🟡 Open — sensor types and mounting TBD |
| HW-08 | 3 | Source/install KWS-303L — grid | 🟡 Open — AC power meter, grid input; RS-485 port 3 |
| HW-09 | 3 | Source/install KWS-303L — generator | 🟡 Open — AC power meter, generator input; RS-485 port 4 |
| HW-10 | 3 | Install GNSS E108-GN03G-485 | 🟡 Open — RS485, IP67; Waveshare port 6 (TCP 4006) |
| HW-12 | — | Replace spare Acurite 5n1 | 🟡 Open — spare confirmed DOA 2026-06-02 |
| HW-13 | 3 | Smart relay for 120VAC load shedding | 🟡 Open — required for OI-24 |
| HW-14 | — | Rain sensor inspection/repair | 🟡 Open — physical inspection at club required |
| HW-15 | 3 | Install POE-SW802-DIN PoE switch | 🟡 Open — powers Waveshare gateway and bay devices |

# 8  Session Log

## 2026-05-27

Full Phase 2 scaffolding complete. Ansible common role live. group_vars/all, host_vars/localhost, site.yml all created and tested. Tasks 1–7 complete.

## 2026-05-28

Full core stack deployed in one session: Mosquitto, InfluxDB, Grafana, rtl_433, WeeWX, nginx, Home Assistant, Pi-hole. Acurite 5n1 (ID 1111) live via rtl_433 → MQTT → WeeWX. Full data chain confirmed.

## 2026-05-29

Phase 2 wrap-up: WeeWX → InfluxDB integration working. nginx reverse proxy live with all .lan domains. Pi-hole DNS operational. MikroTik pinhole configured for external WeeWX access. Grafana RVTC Weather dashboard built.

## 2026-05-30

Backlog formalised. ESPHome ambient sensor YAML built. Grafana RVTC Weather dashboard constructed. Seasons skin CSS cosmetic fix (OI-14).

## 2026-05-31

Waveshare RS485 gateway ordered. mosquitto-clients installed. dvb_usb_rtl28xxu kernel module conflict diagnosed. WiFi IP conflict issue identified. Steve departing for Port Renfrew with RV.

## 2026-06-02  (Port Renfrew — Starlink)

Stack confirmed fully live from campsite over Starlink. Mobile Acurite 5n1 (ID 291, Channel C) decoding. Phase 7 Sensor Fusion architecture scoped.

## 2026-06-03  (Port Renfrew — Starlink)

influxdb2.py unit conversion fix — was writing Fahrenheit as Celsius. InfluxDB bucket flushed for clean data restart. Rain sensor disabled due to phantom trigger investigation.

## 2026-06-05  (Port Renfrew — Starlink)

Rain sensor cumulative fix: contains_total = true applied to rain_mm stanza. Root cause confirmed: Acurite 5n1 sends cumulative bucket counts, not incremental rain. Phantom rain accumulation eliminated.

## 2026-06-07  (Port Renfrew — Starlink)

sqlite3 installed on host. archive_day_rain corruption diagnosed and fixed (rebuild-daily command). Local DNS migrated .local → .lan across all nginx configs, Pi-hole records, and documentation. Avahi/mDNS conflict with .local confirmed.

## 2026-06-09  (Home base — Rogers)

- Returned from Port Renfrew — RV trailering caused WeeWX data gap.

- Full WeeWX / rtl_433 diagnosis session: identified four simultaneous bugs — wrong compose file location (network isolation), USB path hardcoded, invalid MQTT publish topic (+ wildcard), wrong field name mappings (temperature_C/rain_mm vs actual temperature_F/rain_in).

- RTL-SDR Blog V3 (HW-11) received but antenna not yet fitted — could not be commissioned.

- Old clone dongle confirmed working as temporary primary.

- dvb_usb_rtl28xxu blacklist applied and baked into Ansible common role (OI-23).

- OI-22, OI-23, OI-27, OI-17, OI-28, OI-31, OI-26 all closed.

- Belchertown skin installed with dark mode. PyEphem active.

- Grafana user permissions fix applied (472:472 in compose).

- MikroTik RSC updated with ether8 and pinhole fixes.

- PHASE 2 COMPLETE — Phase 3 active.

## 2026-06-10  (Home base — Rogers)

- RTL-SDR Blog V3 (SN 1024, R820T): antenna fitted, confirmed working on Windows (rtl_433 v25.12, ID 1111 decoding), swapped into J45 as primary dongle.

- Clone dongle (SN 00000001, R828D) retained and commissioned as secondary — rtl433b container added to docker-compose.yml.

- Key finding: clone dongle contains R828D tuner (higher spec than assumed), not R820T. Serial numbers confirmed with rtl_eeprom.

- Both containers pin to dongle by serial via -d flag — immune to device index changes on reboot.

- Dual dongle setup confirmed: both decoding Acurite 5n1 ID 1111 independently. Duplicate packets to WeeWX benign.

- docker-compose.yml committed and pushed (85b7357).

- OI-33 added: Club bridge Pi (rtl_433 + WireGuard → home J45 hub).

- OI-34 added: GNSS geofence source inhibit — suppress RV 5n1 when at club.

- Club bridge topology agreed: club Pi → home J45 only; RV always subscribes to home hub regardless of location.

## Next Session — Phase 3 Priorities

- HW-01: Waveshare RS-485 gateway commissioning (device in hand)

- EPEVER MPPT60 Modbus integration (RS-485/1, TCP 4001)

- SAMLUX EVO-2212 inverter-charger Modbus integration (RS-485/2, TCP 4002)

- Solar panel wiring (HW-03 / HW-04)

- Home Assistant onboarding — MQTT integration (OI-15)

- Grafana dashboard rebuild (OI-16)

- WeeWX upstream bug report (OI-32)

# 9  Architecture Notes

## 9.1  Phase 7 — Sensor Fusion

A normalised MQTT sensor bus with a fusion/arbitration layer that assigns the best available source to each logical field, with configurable priority ordering and automatic fallback when a source goes stale.

Planned source types:

- RTL-SDR (dual dongles SN 1024 + SN 00000001) — independent receivers, same 433 MHz sensors

- ESPHome nodes — publish directly to MQTT

- Modbus devices via Waveshare gateway — EPEVER, SAMLUX, KWS-303L, water sensors

- GNSS receiver (HW-10, E108-GN03G-485)

- External APIs — e.g. Environment Canada as fallback weather source

Key design decisions to resolve:

- Normalised MQTT topic schema — proposed: rvtc/sensors/{source_id}/{field}

- Staleness timeout — will differ by source type

- Sanity checking — e.g. reject outTemp = 80°C

- UI approach — live source discovery, per-field priority assignment

- Implementation — Python fusion service (new container + Flask API)

- Duplicate packet handling — deduplicate by (sensor_id, timestamp); arbitrate by signal quality

Consumers: WeeWX (via fused MQTT topic), Home Assistant, Grafana.

## 9.2  Club Bridge Topology (OI-33 / OI-34)

- Small always-on Pi at Solsante Club: rtl_433 + WireGuard.

- Pi connects to home J45 only — never directly to RV.

- Home J45 Mosquitto is the hub — RV subscribes to home regardless of its own location.

- When RV is at club: GNSS geofence (OI-34) detects position match → fusion layer suppresses RV 5n1 (ID 1111) → club station becomes authoritative source.

- When RV is remote: club bridge feeds weather data to home J45 → forwarded to RV via VLAN.

- Reuses OI-20 VPN infrastructure (WireGuard). Prerequisite: HW-10.

## 9.3  Engineering Standards

- Single-point grounding at all instrument enclosures

- Twisted-pair / shielded cable for all analog runs

- 4–20 mA current loop for analog runs exceeding 10 m

- RS-485 Modbus RTU for all digital sensor buses

- TBx-n terminal block wiring convention

- Navy-standard protocols throughout

- All documentation version-controlled in GitHub

## 9.4  SAMLUX Register Map

Full Modbus register map for the SAMLUX EVO-2212 is held locally under NDA. Do not paste register map into any chat session or commit to repository. Reference document must remain offline.