# KWS-303L — Modbus RTU Register Map

RV Total Control Project  ·  Steve Bradshaw (ve7cbh)  ·  June 2026

## 1  Communications Parameters

| Parameter | Value |
| --- | --- |
| Baud rate | 9600 |
| Parity | Even ✓ Confirmed — live commissioning 2026-06-19 (gateway serial config verified Even parity, meter responding correctly) |
| Stop bits | 1 |
| Data bits | 8 |
| Protocol | Modbus RTU — Function Code 03 (read holding registers) / FC 06 (write single register) |
| RS-485 A | Red wire from device RS-485 connector |
| RS-485 B | Black wire from device RS-485 connector |
| Default slave address | 1 (configurable via register 12 or device menu) |
| RVTC addressing note | Use mbpoll with -0 flag for literal register addressing (no Modicon +1 offset). Same convention as SAMLUX EVO-2212. |

## 2  Holding Register Map

All registers are Holding Registers (FC 03 read / FC 06 write). Addresses are literal/direct — no offset. Gain: multiply raw register value by (1/Gain) to obtain engineering units.

| Reg | Description | R/W | Gain | Range / Example | Notes |
| --- | --- | --- | --- | --- | --- |
| Device Identity (Read-Only Fixed Values) |  |  |  |  |  |
| 0 | Connection Command | R | – | – | Purpose unclear — responds on bus scan but value not meaningful to host |
| 1 | Rated Voltage | R | 100 | 22000 = 220 V (fixed) | Factory constant — always 220 V on 303L variant |
| 2 | Rated Current | R | 100 | 4000 = 40 A (fixed) | Factory constant — rated 40 A max |
| 5 | Hardware Version (display board) | R | – | e.g. V0.8 | Lower bits of firmware/hardware version string |
| 12 | Modbus Slave Address | R/W | – | 1–255 | ⚠ Encoded: Modbus value = (address × 256) + 3. Changing this will disconnect the device if the poll config is not also updated. Range 0–255. |
| Real-Time Measurements (Read-Only, Live Values) |  |  |  |  |  |
| 14 | Active Voltage | R | 100 | 22960 = 229.6 V | Primary telemetry — use for shore power present/absent detection |
| 18 | Active Current | R | 1000 | 12 = 0.012 A | Primary telemetry — use for load-shed threshold (OI-24) |
| 26 | Active Power | R | 10 | 5055 = 505.5 W ✓ confirmed | Gain=10 confirmed live: raw 5055 / 10 = 505.5 W, consistent with 119.3 V × 4.571 A × PF. Original source table listed gain=100 (giving 50.55 W — incorrect). ESPHome YAML multiply:0.1 is authoritative. |
| 34 | Unknown | R | ? | Responds on scan | † Value responds on bus scan but purpose is undocumented across all available community sources (Baldin, BieleckiLtd ESPHome repo, community forum). Leave unmapped. |
| 42 | Unknown | R | ? | Responds on scan | † Value responds on bus scan but purpose is undocumented across all available community sources (Baldin, BieleckiLtd ESPHome repo, community forum). Leave unmapped. |
| 48 | Power Factor | R | 1000 | 990 = 0.990 | Dimensionless 0–1. Gain confirmed by ESPHome YAML (multiply: 0.001) |
| 51 | Active Frequency | R | 100 | 5000 = 50.00 Hz |  |
| 55 | Consumed Energy | R | 1000 | 5500 = 5.500 kWh | Cumulative — does not reset on power cycle unless cleared |
| 60 | Internal Temperature | R | 1 | 25 = 25 °C | Sensor is NTC thermistor inside enclosure — not ambient |
| 61 | Timer | R | 1 | 100 = 100 min | Countdown/elapsed timer displayed on unit — purpose TBD |
| 62 | Alarm Code | R | 1 | 0=None, 1=OV, 2=UV, 4=OC, 32=OT | 0=No alarm · 1=Over Voltage · 2=Under Voltage · 4=Over Current · 32=Over Temperature. Cannot be cleared programmatically — only clears when relay is turned on via physical switch. |
| Relay Control (Read/Write) |  |  |  |  |  |
| 63 | Relay State — Switch On/Off | R/W | – | 1 = ON, 0 = OFF | ⚠ R/W — write 1 = ON, 0 = OFF. Unlike the physical switch, turning ON remotely does NOT clear an active alarm. Alarms must be cleared manually. |
| Configurable Limits & Protection Thresholds (Read/Write) |  |  |  |  |  |
| 64 | Over-Voltage Limit | R/W | 10 | 2500 = 250 V (130–290 V) | Alarm triggers when voltage exceeds this value |
| 65 | Under-Voltage Limit | R/W | 10 | 850 = 85 V (85–220 V) | Alarm triggers when voltage falls below this value |
| 66 | Over-Current Limit | R/W | 100 | 260 = 2.6 A (0–40 A) | Alarm triggers when current exceeds this value |
| 67 | Over-Power Limit | R/W | 100 | 110 = 1.1 kW (0–12 kW) | ⚠ Known issue: Over-Power alarm always reports alarm code 0 (no alarm visible in reg 62). Bug in firmware. |
| 72 | Over-Temperature Limit | R/W | 1 | 50 = 50 °C (0–150 °C) | After trip, relay must be manually re-enabled — does NOT auto-restore |
| 73 | Over-Energy Limit (high word) | R/W | – | ×65535 kWh steps | ⚠ Firmware bug: writing reg 73 resets reg 74 to zero (and vice versa). Writing >65.5 kWh requires setting via device menu. Do not write from HA. |
| 74 | Over-Energy Limit (low word) | R/W | 1000 | 1200 = 1.200 kWh (0–65.5 kWh) | ⚠ Firmware bug — see reg 73. Write only to reg 74 to cap at 65.5 kWh. Reading reg 74 alone gives the low-word; full 32-bit value = (reg73 × 65535) + reg74. |
| Display Settings (Read/Write) |  |  |  |  |  |
| 71 | Backlight Timeout | R/W | 1 | 0–60 min (0 = always on) |  |
| 80 | Relay Memory (power-loss behaviour) | R/W | – | 0=Off · 1=On · 2=Hold | Defines relay state after power restoration. 0=Always OFF, 1=Always ON, 2=Hold (restore pre-loss state). |
| 81 | Screen Rotation | R/W | – | 1=Normal · 0=Rotated 180° |  |
| 82 | Backlight Brightness | R/W | 1 | 0–100 % |  |

## 3  RVTC Integration Notes

| Topic | Detail |
| --- | --- |
| Gateway / addressing | Grid meter: RS-485/3, IP 192.168.88.7, TCP 4001, slave 1. Generator meter: RS-485/4, IP 192.168.88.8, TCP 4001, slave 1 (or separate slave IDs if on same bus). |
| mbpoll convention | Always use -0 flag: mbpoll -0 -t 3 -r 14 -c 1 192.168.88.7 4001 (literal addressing — same rule as SAMLUX, see Section 2.9 of Project Reference). |
| OI-24 load-shed registers | Reg 18 (current) on grid meter (RS-485/3) → threshold for grid overload shed. Reg 18 on generator meter (RS-485/4) → threshold for generator overload shed. Reg 14 (voltage) on grid meter → shore power present/absent binary detection. |
| Active Power gain — confirmed | Gain=10 (multiply: 0.1) confirmed live 2026-06-19: raw value 5055 / 10 = 505.5 W, consistent with 119.3 V × 4.571 A. Original source table listed gain=100 (giving 50.55 W — wrong). Use scale: 0.1 in HA modbus YAML for register 26. |
| Registers NOT to map in HA | Regs 34, 42 (unknown — leave unmapped to avoid unnecessary InfluxDB writes). Regs 73, 74 (energy limit — firmware bug; do not write from HA). Reg 12 (slave address — dangerous to write remotely). |
| Parity — confirmed Even | Even parity confirmed live 2026-06-19: mbpoll poll of grid meter (192.168.88.7) returned 119.30 V / 4.571 A / 505.5 W / 60.00 Hz — all consistent with Canadian 120V 60Hz shore power. Gateway serial config is already set to Even. Project Reference comms table should be updated from 'none' to 'even' for both KWS-303L entries. |
| Source / provenance | Register map is community reverse-engineered — manufacturer (kowsi.cn) does not publish it. Primary sources: Alexey Baldin (baldale/kws-303l, March 2025) and BieleckiLtd/KWS-303L-ESPHome YAML (community-confirmed). Registers 34 and 42 remain unidentified in all sources. |

RVTC — KWS-303L Register Map  ·  Community reverse-engineered — not an official manufacturer document  ·  Updated June 2026 — parity and power gain confirmed live against grid meter 192.168.88.7